软件名称: AOI-Vision（AOI-Vision）基于OasysCoreCV的自动光学检测系统软件 版本: V0.2.0 著作权人: 赵达伟 / 潤芯國際(香港)有限公司 / 昆山山之旭电子科技有限公司 源程序前30页（共30页） ======================================================================

// ====== 文件: aoi_engine/init.py ====== 1 | “““AOI Engine — 视觉核心引擎 (独立包) 2 | ======================================== 3 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 4 | 描述: 所有 OasysCoreCV 算法在此包内，与业务层/UI层零耦合。 5 | 可独立于 FastAPI 运行。SysAdmin 可通过 ModuleRegistry 按客户需求打包。 6 | 发布时用 Cython 编译为 .so，保护核心算法。 7 | 8 | 商业化三层架构: 9 | UI Layer (index.html) ←HTTP→ Business Layer (FastAPI) ←import→ Core Engine (aoi_engine) 10 |”“” 11 | from aoi_engine.registry import ModuleRegistry, EngineModule, registry 12 | from aoi_engine.defect import DefectEngine, InspectionResult, Defect 13 | from aoi_engine.vision import VisionAnalyzer 14 | from aoi_engine.camera import BaseCamera, CameraProperties 15 | from aoi_engine.calibration import CalibrationEngine 16 | 17 | all = [ 18 | “DefectEngine”, “VisionAnalyzer”, “BaseCamera”, “CameraProperties”, “CalibrationEngine”, 19 | “ModuleRegistry”, “EngineModule”, “registry”, 20 | “InspectionResult”, “Defect”, 21 | ] 22 | version = “0.2.0”

// ====== 文件: aoi_engine/calibration.py ====== 23 | “““Core Engine: 标定与量测 24 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 25 |”“” 26 | import numpy as np 27 | 28 | class CalibrationEngine: 29 | “““像素标定 + 尺寸量测”“” 30 | 31 | @staticmethod 32 | def calibrate(pixel_distance: float, real_mm: float) -> float: 33 | “““从已知距离计算 pixels_per_mm”“” 34 | if real_mm <= 0: return 0 35 | return pixel_distance / real_mm 36 | 37 | @staticmethod 38 | def measure(pixel_value: float, pixels_per_mm: float) -> dict: 39 | “““像素值转真实尺寸”“” 40 | if pixels_per_mm <= 0: return {“pixel”: pixel_value, “real_mm”: 0} 41 | return {“pixel”: round(pixel_value, 2), “real_mm”: round(pixel_value / pixels_per_mm, 4)}

// ====== 文件: aoi_engine/camera.py ====== 42 | “““Core Engine: 相机抽象 43 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 44 |”“” 45 | from abc import ABC, abstractmethod 46 | from dataclasses import dataclass 47 | import numpy as np 48 | 49 | @dataclass 50 | class CameraProperties:

// ====== Page 1 ======

51 | width: int = 1920; height: int = 1080; fps: float = 30.0 52 | brightness: int = 128; contrast: int = 128; exposure: int = -1 53 | is_open: bool = False; model_name: str = “unknown” 54 | 55 | class BaseCamera(ABC): 56 | @abstractmethod 57 | def open(self, index: int = 0) -> bool: … 58 | @abstractmethod 59 | def read_frame(self) -> np.ndarray | None: … 60 | @abstractmethod 61 | def release(self) -> None: … 62 | @abstractmethod 63 | def get_properties(self) -> CameraProperties: … 64 | def set_property(self, name: str, value: int) -> bool: return False 65 | def enter(self): self.open(); return self 66 | def exit(self, *a): self.release()

// ====== 文件: aoi_engine/defect.py ====== 67 | “““Core Engine: 缺陷检测 68 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 69 | 描述: 8 种 OasysCoreCV 检测模式，独立于业务层。 70 |”“” 71 | import cv2 72 | import numpy as np 73 | from dataclasses import dataclass, field 74 | 75 | @dataclass 76 | class Defect: 77 | type: str = “” 78 | severity: str = “minor” # critical / major / minor 79 | x: int = 0; y: int = 0; w: int = 0; h: int = 0 80 | area: int = 0; description: str = “” 81 | 82 | @dataclass 83 | class InspectionResult: 84 | rule_id: str = “” 85 | defect_count: int = 0 86 | pass_fail: bool = True 87 | defects: list[Defect] = field(default_factory=list) 88 | confidence: float = 1.0 89 | diff_image: np.ndarray | None = None 90 | 91 | class DefectEngine: 92 | “““AOI 缺陷检测引擎 — 8 种模式”“” 93 | 94 | # === 模板比对 === 95 | @staticmethod 96 | def template_match(test_frame: np.ndarray, golden_frame: np.ndarray, threshold: float = 0.95) -> InspectionResult: 97 | gray_t = cv2.cvtColor(test_frame, cv2.COLOR_BGR2GRAY) 98 | gray_g = cv2.cvtColor(golden_frame, cv2.COLOR_BGR2GRAY) 99 | diff = cv2.absdiff(gray_t, gray_g) 100 | _, binary = cv2.threshold(diff, int((1 - threshold) * 255), 255, cv2.THRESH_BINARY)

// ====== Page 2 ======

101 | kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (3, 3)) 102 | cleaned = cv2.morphologyEx(binary, cv2.MORPH_OPEN, kernel) 103 | contours, _ = cv2.findContours(cleaned, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 104 | defects = [] 105 | for c in contours: 106 | area = cv2.contourArea(c) 107 | if area < 50: continue 108 | x, y, w, h = cv2.boundingRect(c) 109 | sev = “critical” if area > 5000 else “major” if area > 1000 else “minor” 110 | defects.append(Defect(type=“diff”, severity=sev, x=int(x), y=int(y), w=int(w), h=int(h), area=int(area), description=f”差异区域 ({int(area)}px²)“)) 111 | return InspectionResult(defect_count=len(defects), pass_fail=len(defects) == 0, defects=defects, confidence=round(1 - len(defects) * 0.05, 2), diff_image=diff) 112 | 113 | # === 特征匹配 (SIFT/ORB) === 114 | @staticmethod 115 | def feature_match(test_frame: np.ndarray, golden_frame: np.ndarray, min_matches: int = 10) -> InspectionResult: 116 | gray_t = cv2.cvtColor(test_frame, cv2.COLOR_BGR2GRAY) 117 | gray_g = cv2.cvtColor(golden_frame, cv2.COLOR_BGR2GRAY) 118 | try: sift = cv2.SIFT_create() 119 | except: sift = cv2.ORB_create(500) 120 | kp1, des1 = sift.detectAndCompute(gray_g, None) 121 | kp2, des2 = sift.detectAndCompute(gray_t, None) 122 | if des1 is None or des2 is None: 123 | return InspectionResult(defect_count=0, pass_fail=True, confidence=0.0) 124 | bf = cv2.BFMatcher() if hasattr(cv2, ‘SIFT_create’) else cv2.BFMatcher(cv2.NORM_HAMMING) 125 | matches = bf.knnMatch(des1, des2, k=2) 126 | good = [m for m, n in matches if m.distance < 0.75 * n.distance] 127 | missing = max(0, len(kp1) - len(good)) 128 | passed = len(good) >= min_matches 129 | defects = [] 130 | if missing > 0: 131 | defects.append(Defect(type=”feature_missing”, severity=“major” if missing > 5 else “minor”, area=missing, description=f”缺失 {missing} 个特征点”)) 132 | return InspectionResult(defect_count=len(defects), pass_fail=passed, defects=defects, confidence=round(len(good) / max(len(kp1), 1), 2)) 133 | 134 | # === 频域缺陷检测 === 135 | @staticmethod 136 | def frequency_domain(test_frame: np.ndarray, threshold: float = 0.1) -> InspectionResult: 137 | gray = cv2.cvtColor(test_frame, cv2.COLOR_BGR2GRAY).astype(np.float32) / 255.0 138 | dft = cv2.dft(gray, flags=cv2.DFT_COMPLEX_OUTPUT) 139 | dft_shift = np.fft.fftshift(dft) 140 | magnitude = cv2.magnitude(dft_shift[:, :, 0], dft_shift[:, :, 1]) 141 | mask = np.ones_like(magnitude) 142 | h, w = magnitude.shape 143 | cv2.circle(mask, (w // 2, h // 2), 30, 0, -1) # 保留低频 144 | masked = magnitude * mask 145 | anomaly_mask = (masked > threshold).astype(np.uint8) * 255 146 | contours, _ = cv2.findContours(anomaly_mask, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 147 | defects = [] 148 | for c in contours: 149 | area = cv2.contourArea(c) 150 | if area < 20: continue

// ====== Page 3 ======

151 | x, y, w, h = cv2.boundingRect(c) 152 | defects.append(Defect(type=“frequency_anomaly”, severity=“major”, x=int(x), y=int(y), w=int(w), h=int(h), area=int(area), description=f”频域异常 ({int(area)}px²)“)) 153 | return InspectionResult(defect_count=len(defects), pass_fail=len(defects) == 0, defects=defects, confidence=round(1 - len(defects) * 0.08, 2)) 154 | 155 | # === 颜色阈值 (HSV) === 156 | @staticmethod 157 | def color_threshold(test_frame: np.ndarray, lower: tuple, upper: tuple) -> InspectionResult: 158 | hsv = cv2.cvtColor(test_frame, cv2.COLOR_BGR2HSV) 159 | mask = cv2.inRange(hsv, lower, upper) 160 | kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (3, 3)) 161 | cleaned = cv2.morphologyEx(mask, cv2.MORPH_OPEN, kernel, iterations=1) 162 | contours, _ = cv2.findContours(cleaned, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 163 | defects = [] 164 | for c in contours: 165 | area = cv2.contourArea(c) 166 | if area < 30: continue 167 | x, y, w, h = cv2.boundingRect(c) 168 | defects.append(Defect(type=”color_deviation”, severity=“major” if area > 2000 else “minor”, x=int(x), y=int(y), w=int(w), h=int(h), area=int(area), description=f”色差区域 ({int(area)}px²)“)) 169 | return InspectionResult(defect_count=len(defects), pass_fail=len(defects) == 0, defects=defects) 170 | 171 | # === 形态学 (顶帽变换) === 172 | @staticmethod 173 | def morphology_tophat(test_frame: np.ndarray, kernel_size: int = 15) -> InspectionResult: 174 | gray = cv2.cvtColor(test_frame, cv2.COLOR_BGR2GRAY) 175 | kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (kernel_size, kernel_size)) 176 | tophat = cv2.morphologyEx(gray, cv2.MORPH_TOPHAT, kernel) 177 | , binary = cv2.threshold(tophat, 30, 255, cv2.THRESH_BINARY) 178 | contours,  = cv2.findContours(binary, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 179 | defects = [] 180 | for c in contours: 181 | area = cv2.contourArea(c) 182 | if area < 10: continue 183 | x, y, w, h = cv2.boundingRect(c) 184 | defects.append(Defect(type=”tophat”, severity=“minor”, x=int(x), y=int(y), w=int(w), h=int(h), area=int(area), description=f”微小凸起/划痕 ({int(area)}px²)“)) 185 | return InspectionResult(defect_count=len(defects), pass_fail=len(defects) == 0, defects=defects) 186 | 187 | # === 轮廓分析 === 188 | @staticmethod 189 | def contour_analysis(test_frame: np.ndarray, min_area: int = 100, max_area: int = 100000) -> InspectionResult: 190 | gray = cv2.cvtColor(test_frame, cv2.COLOR_BGR2GRAY) 191 | blurred = cv2.GaussianBlur(gray, (5, 5), 0) 192 | edges = cv2.Canny(blurred, 50, 150) 193 | contours, _ = cv2.findContours(edges, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 194 | defects = [] 195 | for c in contours: 196 | area = cv2.contourArea(c) 197 | if area < min_area or area > max_area: continue 198 | x, y, w, h = cv2.boundingRect(c) 199 | defects.append(Defect(type=”contour_anomaly”, severity=“minor”, x=int(x), y=int(y), w=int(w), h=int(h), area=int(area), description=f”轮廓异常 ({int(area)}px²)“)) 200 | return InspectionResult(defect_count=len(defects), pass_fail=len(defects) == 0, defects=defects)

// ====== Page 4 ======

201 | 202 | # === 边缘亚像素测量 === 203 | @staticmethod 204 | def subpixel_edge(frame: np.ndarray, roi: tuple) -> list: 205 | x, y, w, h = roi 206 | gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY) 207 | crop = gray[y:y + h, x:x + w] 208 | edges = cv2.Canny(crop, 50, 150) 209 | corners = cv2.goodFeaturesToTrack(edges, 50, 0.01, 10) 210 | if corners is None: return [] 211 | criteria = (cv2.TERM_CRITERIA_EPS + cv2.TERM_CRITERIA_MAX_ITER, 30, 0.001) 212 | refined = cv2.cornerSubPix(crop, corners, (5, 5), (-1, -1), criteria) 213 | return [{“x”: float(p[0][0]) + x, “y”: float(p[0][1]) + y} for p in refined] 214 | 215 | # === 统一入口 === 216 | @staticmethod 217 | def run(frame: np.ndarray, rule: dict, golden_frame: np.ndarray | None = None) -> InspectionResult: 218 | mode = rule.get(“type”, “template_match”) 219 | params = rule.get(“params”, {}) 220 | th = rule.get(“threshold”, 0.95) 221 | if mode == “template_match” and golden_frame is not None: 222 | return DefectEngine.template_match(frame, golden_frame, th) 223 | elif mode == “feature_match” and golden_frame is not None: 224 | return DefectEngine.feature_match(frame, golden_frame, params.get(“min_matches”, 10)) 225 | elif mode == “frequency”: 226 | return DefectEngine.frequency_domain(frame, th) 227 | elif mode == “color_threshold”: 228 | lower = tuple(params.get(“lower”, [0, 0, 0])) 229 | upper = tuple(params.get(“upper”, [180, 255, 255])) 230 | return DefectEngine.color_threshold(frame, lower, upper) 231 | elif mode == “tophat”: 232 | return DefectEngine.morphology_tophat(frame, params.get(“kernel_size”, 15)) 233 | elif mode == “contour”: 234 | return DefectEngine.contour_analysis(frame, params.get(“min_area”, 100), params.get(“max_area”, 100000)) 235 | return DefectEngine.template_match(frame, golden_frame, th) if golden_frame is not None else InspectionResult()

// ====== 文件: aoi_engine/registry.py ====== 236 | “““Core Engine: 模块注册表 — 管理所有 OasysCoreCV 模块的启用/禁用 237 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 238 | 描述: 零依赖纯 Python 注册表。业务层通过 API 控制开关。 239 | 发布时 Cython 编译 .so — 即使服务器被黑也看不到算法逻辑。 240 |”“” 241 | from dataclasses import dataclass, field 242 | from typing import Callable, Any 243 | 244 | 245 | @dataclass 246 | class EngineModule: 247 | “““单个检测模块定义”“” 248 | key: str # 唯一标识 “template_match” / “sift_feature” … 249 | name: str # 中文名 “模板比对” 250 | category: str # 分类: detection / measurement / vision / analysis

// ====== Page 5 ======

251 | description: str # 功能描述 252 | requires: list[str] = field(default_factory=list) # 前置依赖模块 253 | enabled: bool = True # 是否启用 254 | params: dict = field(default_factory=dict) # 模块参数 255 | level: str = “basic” # basic / advanced / enterprise 256 | 257 | 258 | # ====== 全功能模块定义 ====== 259 | ALL_MODULES: list[EngineModule] = [ 260 | # ── 检测类 (Detection) ── 261 | EngineModule(key=“template_match”, name=“模板比对”, category=“detection”, 262 | description=“将待测图像与标准模板逐像素比对，检出差异区域。最简单最快的检测方式。”, 263 | level=“basic”), 264 | EngineModule(key=“feature_match”, name=“特征匹配 (SIFT/ORB)”, category=“detection”, 265 | description=“SIFT/ORB/AKAZE 特征点匹配。旋转/缩放/光照不变，适合定位和缺陷检出。”, 266 | level=“advanced”), 267 | EngineModule(key=“contour_analysis”, name=“轮廓分析”, category=“detection”, 268 | description=“Canny边缘检测+轮廓提取。检查产品外形尺寸/边缘完整性。”, 269 | level=“basic”), 270 | EngineModule(key=“color_threshold”, name=“颜色阈值 (HSV)”, category=“detection”, 271 | description=“HSV色彩空间阈值分割。检出特定颜色区域的异常（污渍/氧化/漏印）。”, 272 | level=“basic”), 273 | EngineModule(key=“frequency_domain”, name=“频域分析 (FFT)”, category=“detection”, 274 | description=“DFT/FFT 频域缺陷检测。周期性纹理缺陷（PCB线路/布料/屏幕像素）的最强手段，频域异常直接暴露。”, 275 | level=“enterprise”), 276 | EngineModule(key=“morphology_tophat”, name=“形态学顶帽”, category=“detection”, 277 | description=“不均匀光照下的微小颗粒/划痕检出。比直接二值化对光照变化鲁棒得多。”, 278 | level=“advanced”), 279 | 280 | # ── 量测类 (Measurement) ── 281 | EngineModule(key=“subpixel_edge”, name=“亚像素边缘定位”, category=“measurement”, 282 | description=“cornerSubPix 亚像素精度角点/边缘定位。μm级精密量测的核心技术。”, 283 | level=“enterprise”), 284 | EngineModule(key=“camera_calibration”, name=“相机标定与畸变校正”, category=“measurement”, 285 | description=“棋盘格标定 → 内参矩阵 + 畸变系数 → 真实世界坐标换算。消除镜头畸变误差。”, 286 | requires=[“subpixel_edge”], level=“enterprise”), 287 | EngineModule(key=“stereo_vision”, name=“立体视觉 (Stereo)”, category=“measurement”, 288 | description=“双相机3D重建。BGA焊点高度/元器件共面度/3D尺寸测量。”, 289 | requires=[“camera_calibration”], level=“enterprise”), 290 | 291 | # ── 视觉增强 (Vision) ── 292 | EngineModule(key=“image_registration”, name=“图像配准 (ECC)”, category=“vision”, 293 | description=“ECC/单应性变换。多相机对齐、抖动补偿、多图叠加去噪。”, 294 | level=“advanced”), 295 | EngineModule(key=“optical_flow”, name=“光流追踪”, category=“vision”, 296 | description=“Farneback/Lucas-Kanade。运动模糊补偿、传送带物体追踪、速度测量。”, 297 | level=“advanced”), 298 | EngineModule(key=“super_resolution”, name=“超分辨率增强”, category=“vision”, 299 | description=“DNN超分模型。低分辨率图像增强细节。对微小缺陷检测有显著提升。”, 300 | requires=[“template_match”], level=“enterprise”),

// ====== Page 6 ======

301 | 302 | # ── DNN 推理 ── 303 | EngineModule(key=“dnn_inference”, name=“DNN 深度学习推理”, category=“vision”, 304 | description=“OasysCoreCV DNN 模块加载 ONNX/TensorFlow/PyTorch 模型。YOLO/MobileNet/ResNet 端侧推理。”, 305 | level=“enterprise”), 306 | EngineModule(key=“object_counting”, name=“目标计数”, category=“vision”, 307 | description=“Blob分析+轮廓计数。零件计数、焊点计数、颗粒计数。”, 308 | level=“basic”), 309 | EngineModule(key=“barcode_reader”, name=“条码/二维码识别”, category=“vision”, 310 | description=“pyzbar 一维码/二维码读取。产品追溯、批次管理。”, 311 | level=“basic”), 312 | 313 | # ── 拼接 ── 314 | EngineModule(key=“image_stitching”, name=“全景拼接”, category=“vision”, 315 | description=“OasysCoreCV Stitcher。大尺寸PCB/晶圆多图拼接。超大工件检测必备。”, 316 | level=“enterprise”), 317 | ] 318 | 319 | 320 | class ModuleRegistry: 321 | “““OasysCoreCV 模块注册表 — 单例 322 |
323 | Usage: 324 | reg = ModuleRegistry() 325 | reg.enable(”feature_match”) # 按客户需求启用 326 | reg.disable(“stereo_vision”) # 按客户预算禁用 327 | reg.enabled_modules # 获取当前启用的模块列表 328 | ““” 329 | _instance = None 330 | 331 | def new(cls): 332 | if cls._instance is None: 333 | cls._instance = super().new(cls) 334 | cls._instance._modules: dict[str, EngineModule] = {} 335 | cls._instance._initialized = False 336 | return cls._instance 337 | 338 | def initialize(self, overrides: dict[str, bool] | None = None): 339 | “““启动时调用，加载所有模块。overrides 可覆盖默认启用状态。”“” 340 | self._modules = {m.key: m for m in ALL_MODULES} 341 | if overrides: 342 | for key, enabled in overrides.items(): 343 | if key in self._modules: 344 | self._modules[key].enabled = enabled 345 | self._initialized = True 346 | 347 | @property 348 | def all_modules(self) -> list[EngineModule]: 349 | return list(self._modules.values()) 350 |

// ====== Page 7 ======

351 | @property 352 | def enabled_modules(self) -> list[EngineModule]: 353 | return [m for m in self._modules.values() if m.enabled] 354 | 355 | @property 356 | def enabled_keys(self) -> list[str]: 357 | return [m.key for m in self.enabled_modules] 358 | 359 | def get(self, key: str) -> EngineModule | None: 360 | return self._modules.get(key) 361 | 362 | def enable(self, key: str) -> bool: 363 | m = self._modules.get(key) 364 | if m: 365 | # Check dependencies 366 | for dep in m.requires: 367 | dep_module = self._modules.get(dep) 368 | if dep_module and not dep_module.enabled: 369 | return False # Dependency not satisfied 370 | m.enabled = True 371 | return True 372 | return False 373 | 374 | def disable(self, key: str) -> bool: 375 | m = self._modules.get(key) 376 | if m: 377 | m.enabled = False 378 | return True 379 | return False 380 | 381 | def toggle(self, key: str, enabled: bool) -> bool: 382 | return self.enable(key) if enabled else self.disable(key) 383 | 384 | def get_config(self) -> dict: 385 | “““导出当前配置（可用于保存/恢复）”“” 386 | return { 387 | “modules”: {m.key: m.enabled for m in self.all_modules}, 388 | “enabled_count”: len(self.enabled_modules), 389 | “total_count”: len(self.all_modules), 390 | “details”: [ 391 | {“key”: m.key, “name”: m.name, “category”: m.category, 392 | “enabled”: m.enabled, “level”: m.level} for m in self.all_modules 393 | ] 394 | } 395 | 396 | def to_dict(self) -> dict: 397 | return self.get_config() 398 | 399 | 400 | # 全局单例

// ====== Page 8 ======

401 | registry = ModuleRegistry() 402 | registry.initialize() # 默认全部启用

// ====== 文件: aoi_engine/setup.py ====== 403 | “““Core Engine: setup.py — Cython 编译支持 404 | ========================================= 405 | 生产部署: 406 | python setup.py build_ext –inplace 407 | → 生成 aoi_engine/defect.cpython-*.so (不可逆, 不可读) 408 | ““” 409 | from setuptools import setup, Extension 410 | from Cython.Build import cythonize 411 | import numpy 412 | 413 | extensions = [ 414 | Extension(“aoi_engine.defect”, [“aoi_engine/defect.py”]), 415 | Extension(“aoi_engine.vision”, [“aoi_engine/vision.py”]), 416 | Extension(“aoi_engine.calibration”, [“aoi_engine/calibration.py”]), 417 | ] 418 | 419 | setup( 420 | name=“aoi-engine”, 421 | version=“0.2.0”, 422 | ext_modules=cythonize(extensions, compiler_directives={“language_level”: “3”}), 423 | include_dirs=[numpy.get_include()], 424 | zip_safe=False, 425 | )

// ====== 文件: aoi_engine/vision.py ====== 426 | “““Core Engine: 视觉分析 (计数+条码+量测) 427 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 428 |”“” 429 | import cv2 430 | import numpy as np 431 | 432 | class VisionAnalyzer: 433 | “““计数 + 条码 + 量测”“” 434 | 435 | @staticmethod 436 | def count_objects(frame: np.ndarray, min_area: int = 100, max_area: int = 100000, threshold: int = 127) -> dict: 437 | gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY) 438 | blurred = cv2.GaussianBlur(gray, (5, 5), 0) 439 | , binary = cv2.threshold(blurred, threshold, 255, cv2.THRESH_BINARY_INV) 440 | kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5)) 441 | cleaned = cv2.morphologyEx(binary, cv2.MORPH_OPEN, kernel) 442 | contours,  = cv2.findContours(cleaned, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) 443 | objects = [] 444 | for c in contours: 445 | area = cv2.contourArea(c) 446 | if min_area <= area <= max_area: 447 | x, y, w, h = cv2.boundingRect(c) 448 | objects.append({“x”: int(x), “y”: int(y), “w”: int(w), “h”: int(h), “area”: int(area)}) 449 | return {“count”: len(objects), “objects”: objects} 450 |

// ====== Page 9 ======

451 | @staticmethod 452 | def read_barcode(frame: np.ndarray) -> list[dict] | None: 453 | try: 454 | from pyzbar.pyzbar import decode 455 | results = decode(frame) 456 | return [{“data”: r.data.decode(“utf-8”), “type”: r.type, “rect”: {“x”: r.rect.left, “y”: r.rect.top, “w”: r.rect.width, “h”: r.rect.height}} for r in results] or None 457 | except ImportError: 458 | return None 459 | 460 | @staticmethod 461 | def measure_distance(p1: tuple, p2: tuple, pixels_per_mm: float) -> dict: 462 | pixel_dist = np.sqrt((p2[0] - p1[0]) ** 2 + (p2[1] - p1[1]) ** 2) 463 | return {“pixel_distance”: round(pixel_dist, 2), “real_mm”: round(pixel_dist / pixels_per_mm, 3)} 464 | 465 | @staticmethod 466 | def measure_circle(center: tuple, radius: int, pixels_per_mm: float) -> dict: 467 | d_px = radius * 2 468 | return {“diameter_px”: round(d_px, 1), “diameter_mm”: round(d_px / pixels_per_mm, 3), 469 | “radius_mm”: round(radius / pixels_per_mm, 3)}

// ====== 文件: backend/alembic/env.py ====== 470 | import asyncio 471 | from logging.config import fileConfig 472 | 473 | from sqlalchemy import pool 474 | from sqlalchemy.engine import Connection 475 | from sqlalchemy.ext.asyncio import async_engine_from_config 476 | 477 | from alembic import context 478 | 479 | # this is the Alembic Config object, which provides 480 | # access to the values within the .ini file in use. 481 | config = context.config 482 | 483 | # Interpret the config file for Python logging. 484 | # This line sets up loggers basically. 485 | if config.config_file_name is not None: 486 | fileConfig(config.config_file_name) 487 | 488 | # add your model’s MetaData object here 489 | # for ‘autogenerate’ support 490 | from app.core.database import Base 491 | from app.modules.auth.models import User, AccountGroup, Permission # noqa: F401 492 | from app.modules.admin.models import FieldOption # noqa: F401 493 | from app.modules.admin.audit_model import AuditLog # noqa: F401 494 | from app.modules.defects.models import InspectionRule, InspectionResult # noqa: F401 495 | from app.modules.measurement.models import Calibration, MeasurementRecord # noqa: F401 496 | target_metadata = Base.metadata 497 | 498 | # other values from the config, defined by the needs of env.py, 499 | # can be acquired: 500 | # my_important_option = config.get_main_option(“my_important_option”)

// ====== Page 10 ======

501 | # … etc. 502 | 503 | 504 | def run_migrations_offline() -> None: 505 | “““Run migrations in ‘offline’ mode. 506 | 507 | This configures the context with just a URL 508 | and not an Engine, though an Engine is acceptable 509 | here as well. By skipping the Engine creation 510 | we don’t even need a DBAPI to be available. 511 | 512 | Calls to context.execute() here emit the given string to the 513 | script output. 514 | 515 |”“” 516 | url = config.get_main_option(“sqlalchemy.url”) 517 | context.configure( 518 | url=url, 519 | target_metadata=target_metadata, 520 | literal_binds=True, 521 | dialect_opts={“paramstyle”: “named”}, 522 | ) 523 | 524 | with context.begin_transaction(): 525 | context.run_migrations() 526 | 527 | 528 | def do_run_migrations(connection: Connection) -> None: 529 | context.configure(connection=connection, target_metadata=target_metadata) 530 | 531 | with context.begin_transaction(): 532 | context.run_migrations() 533 | 534 | 535 | async def run_async_migrations() -> None: 536 | “““In this scenario we need to create an Engine 537 | and associate a connection with the context. 538 | 539 |”“” 540 | 541 | connectable = async_engine_from_config( 542 | config.get_section(config.config_ini_section, {}), 543 | prefix=“sqlalchemy.”, 544 | poolclass=pool.NullPool, 545 | ) 546 | 547 | async with connectable.connect() as connection: 548 | await connection.run_sync(do_run_migrations) 549 | 550 | await connectable.dispose()

// ====== Page 11 ======

551 | 552 | 553 | def run_migrations_online() -> None: 554 | “““Run migrations in ‘online’ mode.”“” 555 | 556 | asyncio.run(run_async_migrations()) 557 | 558 | 559 | if context.is_offline_mode(): 560 | run_migrations_offline() 561 | else: 562 | run_migrations_online()

// ====== 文件: backend/app/init.py ====== 563 | “““AOI-Vision v0.1 564 |”“” 565 | from app.core.database import Base 566 | from app.core.config import settings

// ====== 文件: backend/app/main.py ====== 567 | “““AOI-Vision v0.1 — 自动光学检测系统 568 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 569 | 描述: FastAPI 应用入口，路由注册 570 |”“” 571 | from contextlib import asynccontextmanager 572 | import sys, os 573 | # Add project root to sys.path so aoi_engine can be imported 574 | _project_root = os.path.dirname(os.path.dirname(os.path.abspath(file))) 575 | # file = backend/app/main.py → backend/app → backend → project root 576 | sys.path.insert(0, os.path.dirname(_project_root)) # parent of backend/ = AOI-Vision/ 577 | from fastapi import FastAPI 578 | from fastapi.middleware.cors import CORSMiddleware 579 | from fastapi.staticfiles import StaticFiles 580 | from app.core.database import init_db 581 | from app.modules.auth import router as auth_router 582 | from app.modules.admin import router as admin_router 583 | from app.modules.defects import router as defects_router 584 | from app.modules.measurement import router as measurement_router 585 | from app.modules.reports import router as reports_router 586 | from app.modules.device_router import router as device_router 587 | from app.modules.inspection.websocket import router as ws_router 588 | 589 | @asynccontextmanager 590 | async def lifespan(app: FastAPI): 591 | await init_db() 592 | yield 593 | 594 | app = FastAPI(title=“AOI-Vision”, version=“0.1.0”, lifespan=lifespan) 595 | app.add_middleware(CORSMiddleware, allow_origins=[“*”], allow_methods=[“*”], allow_headers=[“*”]) 596 | app.include_router(auth_router) 597 | app.include_router(admin_router) 598 | app.include_router(defects_router) 599 | app.include_router(measurement_router) 600 | app.include_router(reports_router)

// ====== Page 12 ======

601 | app.include_router(device_router) 602 | app.include_router(ws_router) 603 | 604 | import os 605 | static_path = os.path.join(os.path.dirname(file), “../static”) 606 | if os.path.exists(static_path): 607 | app.mount(“/app”, StaticFiles(directory=static_path, html=True), name=“static”) 608 | 609 | @app.get(“/health”) 610 | async def health(): 611 | return {“status”: “ok”, “version”: “0.1.0”} 612 | 613 | @app.get(“/”) 614 | async def root(): 615 | from fastapi.responses import RedirectResponse 616 | return RedirectResponse(url=“/app/”)

// ====== 文件: backend/app/core/init.py ====== 617 | “““AOI-Vision v0.1 618 |”“”

// ====== 文件: backend/app/core/config.py ====== 619 | “““AOI-Vision v0.1 620 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 621 |”“” 622 | from pydantic_settings import BaseSettings 623 | 624 | class Settings(BaseSettings): 625 | DATABASE_URL: str = “sqlite+aiosqlite:///./aoi_vision.db” 626 | JWT_SECRET_KEY: str = “change-me-in-production” 627 | JWT_ACCESS_TOKEN_EXPIRE_MINUTES: int = 60 628 | UPLOAD_DIR: str = “./uploads” 629 | CAMERA_INDEX: int = 0 630 | CAMERA_TYPE: str = “mock” # mock | uvc | gige | hikvision | … 631 | DEVICE_GATEWAY_TYPE: str = “mock” # mock | haas506 | modbus | opcua | … 632 | 633 | class Config: 634 | env_file = “.env” 635 | 636 | settings = Settings()

// ====== 文件: backend/app/core/database.py ====== 637 | “““AOI-Vision v0.1 638 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 639 | 描述: SQLAlchemy 异步引擎 + SQLite/PostgreSQL 双模 640 |”“” 641 | import uuid as _uuid 642 | from sqlalchemy.ext.asyncio import AsyncSession, async_sessionmaker, create_async_engine 643 | from sqlalchemy.orm import DeclarativeBase 644 | from sqlalchemy.types import TypeDecorator, CHAR 645 | from app.core.config import settings 646 | 647 | class GUID(TypeDecorator): 648 | impl = CHAR; cache_ok = True 649 | def load_dialect_impl(self, dialect): 650 | return dialect.type_descriptor(CHAR(36)) if dialect.name == “sqlite” else dialect.type_descriptor(self.impl)

// ====== Page 13 ======

651 | def process_bind_param(self, value, dialect): 652 | if value is None: return value 653 | return str(value) if dialect.name == “sqlite” else value 654 | def process_result_value(self, value, dialect): 655 | if value is None: return value 656 | return _uuid.UUID(value) if not isinstance(value, _uuid.UUID) else value 657 | 658 | if “sqlite” in settings.DATABASE_URL: 659 | import sqlalchemy.dialects.sqlite as _sq 660 | sq.base.ischema_names[“uuid”] = GUID 661 | 662 | engine = create_async_engine(settings.DATABASE_URL, echo=False) 663 | async_session = async_sessionmaker(engine, class=AsyncSession, expire_on_commit=False) 664 | 665 | if “sqlite” in settings.DATABASE_URL: 666 | from sqlalchemy import event 667 | @event.listens_for(engine.sync_engine, “connect”) 668 | def _sqlite_fk(dbapi_connection, connection_record): 669 | dbapi_connection.execute(“PRAGMA foreign_keys=ON”) 670 | 671 | class Base(DeclarativeBase): pass 672 | 673 | async def get_db(): 674 | async with async_session() as session: 675 | try: 676 | yield session 677 | await session.commit() 678 | except Exception: 679 | await session.rollback() 680 | raise 681 | 682 | async def init_db(): 683 | “““创建表 + 种子数据”“” 684 | async with engine.begin() as conn: 685 | await conn.run_sync(Base.metadata.create_all) 686 |
687 | from app.modules.admin.service import seed_default_options 688 | from app.modules.auth.service import seed_all 689 |
690 | async with async_session() as db: 691 | await seed_default_options(db) 692 | await seed_all(db) 693 | await db.commit()

// ====== 文件: backend/app/core/exceptions.py ====== 694 | “““AOI-Vision v0.1 695 |”“” 696 | from fastapi import HTTPException, status 697 | 698 | class AppException(HTTPException): 699 | def init(self, status_code: int, detail: str): 700 | super().__init__(status_code=status_code, detail=detail)

// ====== Page 14 ======

// ====== 文件: backend/app/core/security.py ====== 701 | “““AOI-Vision v0.1 — 安全模块 702 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 703 | 描述: JWT 令牌生成/验证 + 密码哈希 (bcrypt) 704 |”“” 705 | from datetime import datetime, timedelta, timezone 706 | from jose import JWTError, jwt 707 | import bcrypt 708 | from app.core.config import settings 709 | 710 | def hash_password(password: str) -> str: 711 | return bcrypt.hashpw(password.encode(), bcrypt.gensalt()).decode() 712 | 713 | def verify_password(plain: str, hashed: str) -> bool: 714 | return bcrypt.checkpw(plain.encode(), hashed.encode()) 715 | 716 | def create_access_token(data: dict, expires_minutes: int | None = None) -> str: 717 | expire = datetime.now(timezone.utc) + timedelta(minutes=expires_minutes or settings.JWT_ACCESS_TOKEN_EXPIRE_MINUTES) 718 | to_encode = data.copy() 719 | to_encode.update({“exp”: expire}) 720 | return jwt.encode(to_encode, settings.JWT_SECRET_KEY, algorithm=“HS256”) 721 | 722 | def decode_access_token(token: str) -> dict | None: 723 | try: 724 | return jwt.decode(token, settings.JWT_SECRET_KEY, algorithms=[“HS256”]) 725 | except JWTError: 726 | return None

// ====== 文件: backend/app/hardware/init.py ====== 727 | “““AOI-Vision v0.1 — 硬件层 728 |”“” 729 | from app.hardware.camera import BaseCamera, CameraProperties 730 | from app.hardware.device import BaseDeviceGateway, DeviceTelemetry 731 | from app.hardware.factory import create_camera, create_device_gateway, CAMERA_REGISTRY, DEVICE_REGISTRY 732 | 733 | all = [ 734 | “BaseCamera”, “CameraProperties”, 735 | “BaseDeviceGateway”, “DeviceTelemetry”, 736 | “create_camera”, “create_device_gateway”, 737 | “CAMERA_REGISTRY”, “DEVICE_REGISTRY”, 738 | ]

// ====== 文件: backend/app/hardware/camera.py ====== 739 | “““AOI-Vision v0.1 — 硬件抽象层 740 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 741 | 描述: 摄像头抽象接口 — 所有相机实现必须遵循此协议。 742 | 换硬件只需新增一个子类，不改业务代码。 743 |”“” 744 | from abc import ABC, abstractmethod 745 | from dataclasses import dataclass 746 | from typing import Optional 747 | import numpy as np 748 | 749 | 750 | @dataclass

// ====== Page 15 ======

751 | class CameraProperties: 752 | “““相机属性快照”“” 753 | width: int = 1920 754 | height: int = 1080 755 | fps: float = 30.0 756 | brightness: int = 128 757 | contrast: int = 128 758 | exposure: int = -1 # -1 = auto 759 | is_open: bool = False 760 | model_name: str = “unknown” 761 | 762 | 763 | class BaseCamera(ABC): 764 | “““摄像头抽象基类 — 所有相机驱动必须实现”“” 765 | 766 | @abstractmethod 767 | def open(self, index: int = 0) -> bool: 768 | “““打开摄像头，返回是否成功”“” 769 | … 770 | 771 | @abstractmethod 772 | def read_frame(self) -> Optional[np.ndarray]: 773 | “““读取一帧 (BGR numpy array)，无帧返回 None”“” 774 | … 775 | 776 | @abstractmethod 777 | def release(self) -> None: 778 | “““释放摄像头资源”“” 779 | … 780 | 781 | @abstractmethod 782 | def get_properties(self) -> CameraProperties: 783 | “““获取当前相机属性”“” 784 | … 785 | 786 | def set_property(self, name: str, value: int) -> bool: 787 | “““设置相机属性 (亮度/对比度/曝光)，返回是否成功。 788 | 子类可选覆写。”“” 789 | return False 790 | 791 | def enter(self): 792 | self.open() 793 | return self 794 | 795 | def exit(self, *args): 796 | self.release()

// ====== 文件: backend/app/hardware/device.py ====== 797 | “““AOI-Vision v0.1 — 设备网关抽象层 798 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 799 | 描述: 远程设备网关抽象接口。 800 | 换设备只需新增子类，不改业务代码。

// ====== Page 16 ======

801 | ““” 802 | from abc import ABC, abstractmethod 803 | from dataclasses import dataclass, field 804 | from typing import Optional, Any 805 | from datetime import datetime, timezone 806 | 807 | 808 | @dataclass 809 | class DeviceTelemetry: 810 | “““设备遥测数据”“” 811 | device_id: str = “” 812 | online: bool = False 813 | signal_strength: int = 0 # CSQ 0-31 814 | temperature: float = 0.0 # 设备温度 °C 815 | uptime_seconds: int = 0 816 | relay1_state: bool = False # 继电器1状态 817 | relay2_state: bool = False 818 | gps_lat: float = 0.0 819 | gps_lng: float = 0.0 820 | timestamp: datetime = field(default_factory=lambda: datetime.now(timezone.utc)) 821 | 822 | 823 | class BaseDeviceGateway(ABC): 824 | “““设备网关抽象基类”“” 825 | 826 | @abstractmethod 827 | async def connect(self) -> bool: 828 | “““连接设备，返回是否成功”“” 829 | … 830 | 831 | @abstractmethod 832 | async def get_telemetry(self) -> DeviceTelemetry: 833 | “““获取设备遥测数据”“” 834 | … 835 | 836 | @abstractmethod 837 | async def send_command(self, command: str, params: dict[str, Any] | None = None) -> dict: 838 | “““发送控制指令，返回执行结果”“” 839 | … 840 | 841 | @abstractmethod 842 | async def disconnect(self) -> None: 843 | “““断开连接”“” 844 | …

// ====== 文件: backend/app/hardware/factory.py ====== 845 | “““AOI-Vision v0.1 — 硬件工厂 (懒加载版) 846 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 847 | 描述: 根据配置创建相机和网关实例。 848 | 换硬件只改 config，不碰业务代码。 849 | 驱动模块懒加载——没有 OasysCoreCV 时抽象层仍可 import。 850 |”“”

// ====== Page 17 ======

851 | from typing import Type 852 | from app.hardware.camera import BaseCamera 853 | from app.hardware.device import BaseDeviceGateway 854 | 855 | 856 | CAMERA_REGISTRY: dict[str, str] = { 857 | “uvc”: “app.hardware.uvc_camera.UVCCamera”, 858 | “mock”: “app.hardware.mock_camera.MockCamera”, 859 | # 未来扩展: 860 | # “gige”: “app.hardware.gige_camera.GigECamera”, 861 | # “hikvision”: “app.hardware.hik_camera.HikCamera”, 862 | } 863 | 864 | DEVICE_REGISTRY: dict[str, str] = { 865 | “haas506”: “app.hardware.haas506_gateway.HaaS506Gateway”, 866 | “mock”: “app.hardware.mock_device.MockDeviceGateway”, 867 | # 未来扩展: 868 | # “modbus”: “app.hardware.modbus_gateway.ModbusGateway”, 869 | # “opcua”: “app.hardware.opcua_gateway.OPCUAGateway”, 870 | } 871 | 872 | 873 | def _import_class(dotted_path: str) -> Type: 874 | “““懒加载导入类”“” 875 | module_path, class_name = dotted_path.rsplit(“.”, 1) 876 | import importlib 877 | module = importlib.import_module(module_path) 878 | return getattr(module, class_name) 879 | 880 | 881 | def create_camera(camera_type: str = “mock”, kwargs) -> BaseCamera: 882 | “““根据类型创建相机实例。 883 |
884 | Args: 885 | camera_type: ‘uvc’ | ‘mock’ | ‘gige’ | ‘hikvision’ … 886 | kwargs: 传递给具体相机类 (如 index=0, mode=‘checkerboard’) 887 |
888 | Example: 889 | cam = create_camera(‘mock’, mode=‘checkerboard’, width=1920, height=1080) 890 | cam = create_camera(‘uvc’, index=0) 891 | ““” 892 | dotted = CAMERA_REGISTRY.get(camera_type) 893 | if dotted is None: 894 | raise ValueError(f”Unknown camera type: {camera_type}. Available: {list(CAMERA_REGISTRY.keys())}“) 895 | cls = _import_class(dotted) 896 | return cls(kwargs) 897 | 898 | 899 | def create_device_gateway(gateway_type: str = “mock”, kwargs) -> BaseDeviceGateway: 900 | “““根据类型创建设备网关实例。

// ====== Page 18 ======

901 |
902 | Args: 903 | gateway_type: ‘haas506’ | ‘mock’ | ‘modbus’ | ‘opcua’ … 904 | **kwargs: device_id, product_key, … 905 |
906 | Example: 907 | gw = create_device_gateway(‘mock’, device_id=‘test-001’) 908 | gw = create_device_gateway(‘haas506’, device_id=‘hd3-001’, product_key=‘…’) 909 | ““” 910 | dotted = DEVICE_REGISTRY.get(gateway_type) 911 | if dotted is None: 912 | raise ValueError(f”Unknown device type: {gateway_type}. Available: {list(DEVICE_REGISTRY.keys())}“) 913 | cls = _import_class(dotted) 914 | return cls(**kwargs)

// ====== 文件: backend/app/hardware/haas506_gateway.py ====== 915 | “““AOI-Vision v0.1 — HaaS506 设备网关 916 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 917 | 描述: 通过 MQTT/阿里云 IoT 与 HaaS506-HD3 通信。 918 | 远程获取遥测、控制继电器、接收告警。 919 |”“” 920 | import json 921 | import asyncio 922 | from typing import Optional, Any 923 | from datetime import datetime, timezone 924 | from app.hardware.device import BaseDeviceGateway, DeviceTelemetry 925 | 926 | 927 | class HaaS506Gateway(BaseDeviceGateway): 928 | “““HaaS506-HD3 4G RTU 设备网关 (MQTT)”“” 929 | 930 | def init(self, device_id: str, product_key: str = ““, 931 | device_name: str =”“, device_secret: str =”“, 932 | region: str =”cn-shanghai”): 933 | self.device_id = device_id 934 | self._product_key = product_key 935 | self._device_name = device_name 936 | self._device_secret = device_secret 937 | self._region = region 938 | self._connected = False 939 | self._last_telemetry: Optional[DeviceTelemetry] = None 940 | 941 | async def connect(self) -> bool: 942 | “““连接 HaaS506，通过阿里云 IoT MQTT”“” 943 | # TODO: 实际实现使用 aliyunIoT SDK 或 paho-mqtt 944 | # 现阶段返回模拟连接状态 945 | self._connected = True 946 | return True 947 | 948 | async def get_telemetry(self) -> DeviceTelemetry: 949 | “““从 HaaS506 拉取最新遥测”“” 950 | if self._last_telemetry:

// ====== Page 19 ======

951 | self._last_telemetry.timestamp = datetime.now(timezone.utc) 952 | return self._last_telemetry 953 | # 模拟遥测 954 | return DeviceTelemetry( 955 | device_id=self.device_id, 956 | online=self._connected, 957 | signal_strength=25, 958 | temperature=42.5, 959 | uptime_seconds=3600, 960 | relay1_state=False, 961 | relay2_state=False, 962 | ) 963 | 964 | async def send_command(self, command: str, params: dict[str, Any] | None = None) -> dict: 965 | “““通过 MQTT 下发指令到 HaaS506”“” 966 | if not self._connected: 967 | return {“success”: False, “error”: “Device not connected”} 968 | # TODO: 实际 MQTT publish 969 | if command == “set_relay”: 970 | relay_id = params.get(“relay”, 1) if params else 1 971 | state = params.get(“state”, False) if params else False 972 | return {“success”: True, “relay”: relay_id, “state”: state} 973 | elif command == “reboot”: 974 | return {“success”: True, “action”: “reboot_scheduled”} 975 | return {“success”: False, “error”: f”Unknown command: {command}“} 976 | 977 | async def disconnect(self) -> None: 978 | self._connected = False

// ====== 文件: backend/app/hardware/mock_camera.py ====== 979 | “““AOI-Vision v0.1 — Mock 相机 980 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 981 | 描述: 无硬件时的模拟相机 — 用于开发测试/CI。 982 | 支持纯色图、棋盘格、从图片文件读取。 983 |”“” 984 | import cv2 985 | import numpy as np 986 | from typing import Optional 987 | from app.hardware.camera import BaseCamera, CameraProperties 988 | 989 | 990 | class MockCamera(BaseCamera): 991 | “““模拟相机 — 无硬件依赖，用于开发和自动化测试”“” 992 | 993 | MODE_SOLID = “solid” 994 | MODE_CHECKERBOARD = “checkerboard” 995 | MODE_FILE = “file” 996 | 997 | def init(self, mode: str = MODE_CHECKERBOARD, 998 | width: int = 1920, height: int = 1080, 999 | file_path: Optional[str] = None): 1000 | self._mode = mode

// ====== Page 20 ======

1001 | self._file_path = file_path 1002 | self._frame_cache: Optional[np.ndarray] = None 1003 | self._props = CameraProperties( 1004 | width=width, height=height, fps=30, 1005 | model_name=f”Mock Camera ({mode})” 1006 | ) 1007 | 1008 | def open(self, index: int = 0) -> bool: 1009 | if self._mode == self.MODE_FILE and self._file_path: 1010 | img = cv2.imread(self._file_path) 1011 | if img is not None: 1012 | self._frame_cache = cv2.resize(img, (self._props.width, self._props.height)) 1013 | if self._mode == self.MODE_SOLID: 1014 | self._frame_cache = np.ones( 1015 | (self._props.height, self._props.width, 3), dtype=np.uint8) * 128 1016 | self._props.is_open = True 1017 | return True 1018 | 1019 | def read_frame(self) -> Optional[np.ndarray]: 1020 | if not self._props.is_open: 1021 | return None 1022 | if self._frame_cache is not None: 1023 | return self._frame_cache.copy() 1024 | # 动态生成棋盘格 1025 | size = 100 1026 | rows = self._props.height // size + 1 1027 | cols = self._props.width // size + 1 1028 | frame = np.zeros((self._props.height, self._props.width, 3), dtype=np.uint8) 1029 | for r in range(rows): 1030 | for c in range(cols): 1031 | if (r + c) % 2 == 0: 1032 | y1, y2 = r * size, min((r + 1) * size, self._props.height) 1033 | x1, x2 = c * size, min((c + 1) * size, self._props.width) 1034 | frame[y1:y2, x1:x2] = 200 1035 | return frame 1036 | 1037 | def release(self) -> None: 1038 | self._props.is_open = False 1039 | 1040 | def get_properties(self) -> CameraProperties: 1041 | return self._props

// ====== 文件: backend/app/hardware/mock_device.py ====== 1042 | “““AOI-Vision v0.1 — Mock 设备网关 1043 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1044 | 描述: 无硬件时的模拟设备网关，用于开发测试。 1045 |”“” 1046 | import random 1047 | from typing import Any 1048 | from datetime import datetime, timezone 1049 | from app.hardware.device import BaseDeviceGateway, DeviceTelemetry 1050 |

// ====== Page 21 ======

1051 | 1052 | class MockDeviceGateway(BaseDeviceGateway): 1053 | “““模拟设备网关 — 无硬件依赖”“” 1054 | 1055 | def init(self, device_id: str = “mock-device-001”): 1056 | self.device_id = device_id 1057 | self._connected = False 1058 | self._relay1 = False 1059 | self._relay2 = False 1060 | self._start_time = datetime.now(timezone.utc) 1061 | 1062 | async def connect(self) -> bool: 1063 | self._connected = True 1064 | return True 1065 | 1066 | async def get_telemetry(self) -> DeviceTelemetry: 1067 | uptime = int((datetime.now(timezone.utc) - self._start_time).total_seconds()) 1068 | return DeviceTelemetry( 1069 | device_id=self.device_id, 1070 | online=self._connected, 1071 | signal_strength=random.randint(20, 31), 1072 | temperature=round(random.uniform(35, 50), 1), 1073 | uptime_seconds=uptime, 1074 | relay1_state=self._relay1, 1075 | relay2_state=self._relay2, 1076 | gps_lat=22.5431 + random.uniform(-0.01, 0.01), 1077 | gps_lng=113.929 + random.uniform(-0.01, 0.01), 1078 | ) 1079 | 1080 | async def send_command(self, command: str, params: dict[str, Any] | None = None) -> dict: 1081 | if not self._connected: 1082 | return {“success”: False, “error”: “Not connected”} 1083 | if command == “set_relay”: 1084 | relay_id = params.get(“relay”, 1) if params else 1 1085 | state = params.get(“state”, False) if params else False 1086 | if relay_id == 1: 1087 | self._relay1 = state 1088 | else: 1089 | self._relay2 = state 1090 | return {“success”: True} 1091 | return {“success”: False, “error”: f”Unknown: {command}“} 1092 | 1093 | async def disconnect(self) -> None: 1094 | self._connected = False

// ====== 文件: backend/app/hardware/uvc_camera.py ====== 1095 | “““AOI-Vision v0.1 — UVC 相机驱动 1096 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1097 | 描述: 通用 USB Video Class 摄像头实现。 1098 | 兼容龙仕显微镜摄像头及所有标准 UVC 设备。 1099 |”“” 1100 | import cv2

// ====== Page 22 ======

1101 | import numpy as np 1102 | from typing import Optional 1103 | from app.hardware.camera import BaseCamera, CameraProperties 1104 | 1105 | 1106 | class UVCCamera(BaseCamera): 1107 | “““通用 USB 相机 — 基于 OasysCoreCV VideoCapture”“” 1108 | 1109 | def init(self): 1110 | self._cap: Optional[cv2.VideoCapture] = None 1111 | self._props = CameraProperties(model_name=“UVC Camera”) 1112 | 1113 | def open(self, index: int = 0) -> bool: 1114 | self._cap = cv2.VideoCapture(index) 1115 | if not self._cap.isOpened(): 1116 | self._cap = None 1117 | return False 1118 | # 尝试读取实际参数 1119 | w = int(self._cap.get(cv2.CAP_PROP_FRAME_WIDTH)) 1120 | h = int(self._cap.get(cv2.CAP_PROP_FRAME_HEIGHT)) 1121 | fps = self._cap.get(cv2.CAP_PROP_FPS) 1122 | if w > 0 and h > 0: 1123 | self._props.width = w 1124 | self._props.height = h 1125 | if fps > 0: 1126 | self._props.fps = fps 1127 | self._props.is_open = True 1128 | return True 1129 | 1130 | def read_frame(self) -> Optional[np.ndarray]: 1131 | if self._cap is None: 1132 | return None 1133 | ret, frame = self._cap.read() 1134 | return frame if ret else None 1135 | 1136 | def release(self) -> None: 1137 | if self._cap: 1138 | self._cap.release() 1139 | self._cap = None 1140 | self._props.is_open = False 1141 | 1142 | def get_properties(self) -> CameraProperties: 1143 | return self._props 1144 | 1145 | def set_property(self, name: str, value: int) -> bool: 1146 | if self._cap is None: 1147 | return False 1148 | prop_map = { 1149 | “brightness”: cv2.CAP_PROP_BRIGHTNESS, 1150 | “contrast”: cv2.CAP_PROP_CONTRAST,

// ====== Page 23 ======

1151 | “exposure”: cv2.CAP_PROP_EXPOSURE, 1152 | “width”: cv2.CAP_PROP_FRAME_WIDTH, 1153 | “height”: cv2.CAP_PROP_FRAME_HEIGHT, 1154 | } 1155 | prop_id = prop_map.get(name) 1156 | if prop_id is None: 1157 | return False 1158 | self._cap.set(prop_id, value) 1159 | return True

// ====== 文件: backend/app/modules/device_router.py ====== 1160 | “““AOI-Vision v0.1 — 设备网关路由 1161 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1162 | 描述: 设备管理 API — HaaS506 遥测查看 + 远程控制 1163 |”“” 1164 | from fastapi import APIRouter, Depends 1165 | from app.core.config import settings 1166 | from app.modules.auth.service import get_current_user 1167 | from app.hardware import create_device_gateway 1168 | 1169 | router = APIRouter(prefix=“/device”, tags=[“Device”]) 1170 | 1171 | @router.get(“/telemetry”) 1172 | async def device_telemetry(_user=Depends(get_current_user)): 1173 | gw = create_device_gateway(settings.DEVICE_GATEWAY_TYPE) 1174 | await gw.connect() 1175 | try: 1176 | telemetry = await gw.get_telemetry() 1177 | return telemetry 1178 | finally: 1179 | await gw.disconnect() 1180 | 1181 | @router.post(“/relay”) 1182 | async def set_relay(relay: int = 1, state: bool = False, _user=Depends(get_current_user)): 1183 | from app.modules.auth.service import require_permission 1184 | _admin = require_permission(“device”, “manage”) 1185 | gw = create_device_gateway(settings.DEVICE_GATEWAY_TYPE) 1186 | await gw.connect() 1187 | try: 1188 | result = await gw.send_command(“set_relay”, {“relay”: relay, “state”: state}) 1189 | return result 1190 | finally: 1191 | await gw.disconnect()

// ====== 文件: backend/app/modules/admin/init.py ====== 1192 | “““AOI-Vision v0.1 1193 |”“” 1194 | from app.modules.admin.router import router 1195 | from app.modules.admin.service import seed_default_options 1196 | 1197 | all = [“router”, “seed_default_options”]

// ====== 文件: backend/app/modules/admin/audit_model.py ====== 1198 | “““AOI-Vision v0.1 — 审计日志模型 1199 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1200 | 描述: ERPNext 风格审计 — CUD 操作全记录

// ====== Page 24 ======

1201 | ““” 1202 | import uuid 1203 | from datetime import datetime, timezone 1204 | from sqlalchemy import DateTime, String, Text, func 1205 | from app.core.database import GUID 1206 | from sqlalchemy.dialects.sqlite import JSON as SQLiteJSON 1207 | from sqlalchemy.orm import Mapped, mapped_column 1208 | from app.core.config import settings 1209 | from app.core.database import Base 1210 | 1211 | # SQLite 用 JSON，PostgreSQL 用 JSONB 1212 | JSONType = SQLiteJSON if “sqlite” in settings.DATABASE_URL else import(“sqlalchemy.dialects.postgresql”, fromlist=[“JSONB”]).JSONB 1213 | 1214 | class AuditLog(Base): 1215 | tablename = “audit_logs” 1216 | id: Mapped[uuid.UUID] = mapped_column(GUID, primary_key=True, default=uuid.uuid4) 1217 | timestamp: Mapped[datetime] = mapped_column(DateTime(timezone=True), default=lambda: datetime.now(timezone.utc), server_default=func.now(), index=True) 1218 | username: Mapped[str] = mapped_column(String(100), nullable=False, default=“system”) 1219 | action: Mapped[str] = mapped_column(String(20), nullable=False, index=True) 1220 | entity_type: Mapped[str] = mapped_column(String(100), nullable=False, index=True) 1221 | entity_id: Mapped[str | None] = mapped_column(String(100), nullable=True) 1222 | entity_label: Mapped[str | None] = mapped_column(String(200), nullable=True) 1223 | changes: Mapped[dict | None] = mapped_column(JSONType, nullable=True) 1224 | summary: Mapped[str | None] = mapped_column(Text, nullable=True)

// ====== 文件: backend/app/modules/admin/models.py ====== 1225 | “““AOI-Vision v0.1 — 参数枚举模块 1226 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1227 | 描述: FieldOption 模型 — 可配置的下拉选项（缺陷类型/产品型号/产线等） 1228 |”“” 1229 | from sqlalchemy import Boolean, Integer, String 1230 | from sqlalchemy.orm import Mapped, mapped_column 1231 | from app.core.database import Base 1232 | 1233 | class FieldOption(Base): 1234 | tablename = “field_options” 1235 | id: Mapped[int] = mapped_column(Integer, primary_key=True, autoincrement=True) 1236 | field_name: Mapped[str] = mapped_column(String(100), nullable=False, index=True) 1237 | option_value: Mapped[str] = mapped_column(String(200), nullable=False) 1238 | option_label: Mapped[str] = mapped_column(String(200), nullable=False) 1239 | sort_order: Mapped[int] = mapped_column(Integer, default=0) 1240 | is_active: Mapped[bool] = mapped_column(Boolean, default=True)

// ====== 文件: backend/app/modules/admin/router.py ====== 1241 | “““AOI-Vision v0.1 — 管理路由 1242 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1243 |”“” 1244 | from fastapi import APIRouter, Depends, Query 1245 | from sqlalchemy.ext.asyncio import AsyncSession 1246 | from app.core.database import get_db 1247 | from app.modules.auth.service import get_current_user, require_permission 1248 | from app.modules.admin.schemas import FieldOptionCreate, FieldOptionUpdate, FieldOptionResponse, AuditLogListResponse 1249 | from app.modules.admin.service import list_options, create_option, update_option, delete_option, query_audit_logs 1250 |

// ====== Page 25 ======

1251 | router = APIRouter(prefix=“/admin”, tags=[“Admin”]) 1252 | 1253 | @router.get(“/field-options”, response_model=list[FieldOptionResponse]) 1254 | async def get_options(field_name: str | None = Query(None), db: AsyncSession = Depends(get_db), _user=Depends(get_current_user)): 1255 | return await list_options(db, field_name) 1256 | 1257 | @router.post(“/field-options”, response_model=FieldOptionResponse) 1258 | async def add_option(data: FieldOptionCreate, db: AsyncSession = Depends(get_db), _admin=require_permission(“admin”, “manage”)): 1259 | return await create_option(db, data) 1260 | 1261 | @router.put(“/field-options/{opt_id}”, response_model=FieldOptionResponse) 1262 | async def edit_option(opt_id: int, data: FieldOptionUpdate, db: AsyncSession = Depends(get_db), _admin=require_permission(“admin”, “manage”)): 1263 | opt = await update_option(db, opt_id, data) 1264 | if not opt: 1265 | from app.core.exceptions import AppException 1266 | raise AppException(404, “Option not found”) 1267 | return opt 1268 | 1269 | @router.delete(“/field-options/{opt_id}”, status_code=204) 1270 | async def remove_option(opt_id: int, db: AsyncSession = Depends(get_db), _admin=require_permission(“admin”, “manage”)): 1271 | ok = await delete_option(db, opt_id) 1272 | if not ok: 1273 | from app.core.exceptions import AppException 1274 | raise AppException(404, “Option not found”) 1275 | 1276 | # ====== 模块管理 ====== 1277 | 1278 | @router.get(“/engine-modules”) 1279 | async def get_modules(_user=Depends(get_current_user)): 1280 | “““查看所有 OasysCoreCV 模块及启用状态”“” 1281 | from aoi_engine import registry 1282 | return registry.get_config() 1283 | 1284 | @router.post(“/engine-modules/{module_key}”) 1285 | async def toggle_module(module_key: str, data: dict, _admin=require_permission(“admin”, “manage”)): 1286 | “““启用/禁用指定模块”“” 1287 | from aoi_engine import registry 1288 | enabled = data.get(“enabled”, True) 1289 | ok = registry.toggle(module_key, enabled) 1290 | return {“key”: module_key, “enabled”: registry.get(module_key).enabled if registry.get(module_key) else None, “success”: ok} 1291 | 1292 | @router.get(“/engine-modules/config”) 1293 | async def export_module_config(_user=Depends(get_current_user)): 1294 | “““导出当前模块配置快照（用于客户打包）”“” 1295 | from aoi_engine import registry 1296 | return registry.get_config() 1297 | 1298 | @router.get(“/audit-logs”, response_model=AuditLogListResponse) 1299 | async def get_audit_logs(entity_type: str | None = Query(None), action: str | None = Query(None), 1300 | username: str | None = Query(None), limit: int = Query(50, ge=1, le=200),

// ====== Page 26 ======

1301 | offset: int = Query(0, ge=0), db: AsyncSession = Depends(get_db), _user=Depends(get_current_user)): 1302 | items, total = await query_audit_logs(db, entity_type, action, username, limit, offset) 1303 | return {“items”: items, “total”: total, “limit”: limit, “offset”: offset}

// ====== 文件: backend/app/modules/admin/schemas.py ====== 1304 | “““AOI-Vision v0.1 — 管理模块 schemas 1305 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1306 |”“” 1307 | from pydantic import BaseModel 1308 | from datetime import datetime 1309 | 1310 | class FieldOptionCreate(BaseModel): 1311 | field_name: str 1312 | option_value: str 1313 | option_label: str 1314 | sort_order: int = 0 1315 | 1316 | class FieldOptionUpdate(BaseModel): 1317 | option_value: str | None = None 1318 | option_label: str | None = None 1319 | sort_order: int | None = None 1320 | is_active: bool | None = None 1321 | 1322 | class FieldOptionResponse(BaseModel): 1323 | id: int 1324 | field_name: str 1325 | option_value: str 1326 | option_label: str 1327 | sort_order: int 1328 | is_active: bool 1329 | model_config = {“from_attributes”: True} 1330 | 1331 | class AuditLogResponse(BaseModel): 1332 | id: str 1333 | timestamp: datetime 1334 | username: str 1335 | action: str 1336 | entity_type: str 1337 | entity_label: str | None = None 1338 | summary: str | None = None 1339 | model_config = {“from_attributes”: True} 1340 | 1341 | class AuditLogListResponse(BaseModel): 1342 | items: list[AuditLogResponse] 1343 | total: int 1344 | limit: int 1345 | offset: int

// ====== 文件: backend/app/modules/admin/service.py ====== 1346 | “““AOI-Vision v0.1 — 管理服务 1347 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1348 |”“” 1349 | from sqlalchemy import select, desc, func 1350 | from sqlalchemy.ext.asyncio import AsyncSession

// ====== Page 27 ======

1351 | from app.modules.admin.models import FieldOption 1352 | from app.modules.admin.audit_model import AuditLog 1353 | from app.modules.admin.schemas import FieldOptionCreate, FieldOptionUpdate 1354 | 1355 | async def list_options(db: AsyncSession, field_name: str | None = None) -> list[FieldOption]: 1356 | stmt = select(FieldOption).where(FieldOption.is_active == True).order_by(FieldOption.sort_order) 1357 | if field_name: 1358 | stmt = stmt.where(FieldOption.field_name == field_name) 1359 | return list((await db.execute(stmt)).scalars().all()) 1360 | 1361 | async def create_option(db: AsyncSession, data: FieldOptionCreate) -> FieldOption: 1362 | opt = FieldOption(**data.model_dump()) 1363 | db.add(opt) 1364 | await db.flush() 1365 | return opt 1366 | 1367 | async def update_option(db: AsyncSession, opt_id: int, data: FieldOptionUpdate) -> FieldOption | None: 1368 | result = await db.execute(select(FieldOption).where(FieldOption.id == opt_id)) 1369 | opt = result.scalar_one_or_none() 1370 | if not opt: 1371 | return None 1372 | for key, val in data.model_dump(exclude_none=True).items(): 1373 | setattr(opt, key, val) 1374 | await db.flush() 1375 | return opt 1376 | 1377 | async def delete_option(db: AsyncSession, opt_id: int) -> bool: 1378 | result = await db.execute(select(FieldOption).where(FieldOption.id == opt_id)) 1379 | opt = result.scalar_one_or_none() 1380 | if not opt: 1381 | return False 1382 | opt.is_active = False 1383 | await db.flush() 1384 | return True 1385 | 1386 | async def seed_default_options(db: AsyncSession): 1387 | “““初始化默认枚举值”“” 1388 | defaults = [ 1389 | (“defect_type”, “scratch”, “划痕”, 1), 1390 | (“defect_type”, “dent”, “凹陷”, 2), 1391 | (“defect_type”, “stain”, “污渍”, 3), 1392 | (“defect_type”, “crack”, “裂纹”, 4), 1393 | (“defect_type”, “burr”, “毛刺”, 5), 1394 | (“defect_severity”, “critical”, “严重”, 1), 1395 | (“defect_severity”, “major”, “主要”, 2), 1396 | (“defect_severity”, “minor”, “次要”, 3), 1397 | (“product_model”, “model-a”, “产品A型”, 1), 1398 | (“product_model”, “model-b”, “产品B型”, 2), 1399 | (“production_line”, “line-1”, “产线1号”, 1), 1400 | (“production_line”, “line-2”, “产线2号”, 2),

// ====== Page 28 ======

1401 | ] 1402 | for fn, ov, ol, so in defaults: 1403 | existing = await db.execute(select(FieldOption).where(FieldOption.field_name == fn, FieldOption.option_value == ov)) 1404 | if not existing.scalar_one_or_none(): 1405 | db.add(FieldOption(field_name=fn, option_value=ov, option_label=ol, sort_order=so)) 1406 | await db.flush() 1407 | 1408 | async def query_audit_logs(db: AsyncSession, entity_type: str | None = None, action: str | None = None, 1409 | username: str | None = None, limit: int = 50, offset: int = 0) -> tuple[list, int]: 1410 | stmt = select(AuditLog) 1411 | count_stmt = select(func.count()).select_from(AuditLog) 1412 | if entity_type: 1413 | stmt = stmt.where(AuditLog.entity_type == entity_type) 1414 | count_stmt = count_stmt.where(AuditLog.entity_type == entity_type) 1415 | if action: 1416 | stmt = stmt.where(AuditLog.action == action) 1417 | count_stmt = count_stmt.where(AuditLog.action == action) 1418 | if username: 1419 | stmt = stmt.where(AuditLog.username == username) 1420 | count_stmt = count_stmt.where(AuditLog.username == username) 1421 | stmt = stmt.order_by(desc(AuditLog.timestamp)).offset(offset).limit(limit) 1422 | items = list((await db.execute(stmt)).scalars().all()) 1423 | total = (await db.execute(count_stmt)).scalar() or 0 1424 | return items, total

// ====== 文件: backend/app/modules/auth/init.py ====== 1425 | “““AOI-Vision v0.1 1426 |”“” 1427 | from app.modules.auth.router import router 1428 | 1429 | all = [“router”]

// ====== 文件: backend/app/modules/auth/models.py ====== 1430 | “““AOI-Vision v0.2 — RBAC 权限模型 1431 | 日期: 2026-06-17 | 作者: William Chao / OASYS CORE 1432 | 描述: 全多对多 RBAC — 权限组 ↔ 账号组 ↔ 用户 1433 | Permission Group (权限组) ←M2M→ Account Group (账号组) ←M2M→ User (用户) 1434 |”“” 1435 | import uuid 1436 | from sqlalchemy import Boolean, ForeignKey, String, Table, Column, JSON 1437 | from sqlalchemy.orm import Mapped, mapped_column, relationship 1438 | from app.core.database import Base, GUID 1439 | from app.shared.models import TimestampMixin 1440 | 1441 | # ====== M2M 关联表 ====== 1442 | 1443 | user_account_groups = Table( 1444 | “user_account_groups”, Base.metadata, 1445 | Column(“user_id”, GUID, ForeignKey(“users.id”, ondelete=“CASCADE”), primary_key=True), 1446 | Column(“account_group_id”, GUID, ForeignKey(“account_groups.id”, ondelete=“CASCADE”), primary_key=True), 1447 | ) 1448 | 1449 | account_group_permissions = Table( 1450 | “account_group_permissions”, Base.metadata,

// ====== Page 29 ======

1451 | Column(“account_group_id”, GUID, ForeignKey(“account_groups.id”, ondelete=“CASCADE”), primary_key=True), 1452 | Column(“permission_id”, GUID, ForeignKey(“permissions.id”, ondelete=“CASCADE”), primary_key=True), 1453 | ) 1454 | 1455 | 1456 | # ====== 权限组 ====== 1457 | 1458 | class Permission(Base): 1459 | “““权限组 — 定义一组操作权限”“” 1460 | tablename = “permissions” 1461 | 1462 | id: Mapped[uuid.UUID] = mapped_column(GUID, primary_key=True, default=uuid.uuid4) 1463 | name: Mapped[str] = mapped_column(String(100), unique=True, nullable=False) 1464 | description: Mapped[str] = mapped_column(String(500), default=““) 1465 | # permissions dict: {”inspection”:“manage”,“measurement”:“read”,“defects”:“manage”,…} 1466 | permissions: Mapped[dict] = mapped_column(JSON, default=dict) 1467 | is_active: Mapped[bool] = mapped_column(Boolean, default=True) 1468 | 1469 | account_groups: Mapped[list[“AccountGroup”]] = relationship( 1470 | “AccountGroup”, secondary=account_group_permissions, back_populates=“permissions” 1471 | ) 1472 | 1473 | 1474 | # ====== 账号组 ====== 1475 | 1476 | class AccountGroup(Base): 1477 | “““账号组 — 用户逻辑分组（如：质检员组、管理员组、操作员组）”“” 1478 | tablename = “account_groups” 1479 | 1480 | id: Mapped[uuid.UUID] = mapped_column(GUID, primary_key=True, default=uuid.uuid4) 1481 | name: Mapped[str] = mapped_column(String(100), unique=True, nullable=False) 1482 | description: Mapped[str] = mapped_column(String(500), default=““) 1483 | is_active: Mapped[bool] = mapped_column(Boolean, default=True) 1484 | 1485 | users: Mapped[list[”User”]] = relationship( 1486 |”User”, secondary=user_account_groups, back_populates=“account_groups” 1487 | ) 1488 | permissions: Mapped[list[“Permission”]] = relationship( 1489 | “Permission”, secondary=account_group_permissions, back_populates=“account_groups” 1490 | ) 1491 | 1492 | 1493 | # ====== 用户 ====== 1494 | 1495 | class User(TimestampMixin, Base): 1496 | tablename = “users” 1497 | 1498 | id: Mapped[uuid.UUID] = mapped_column(GUID, primary_key=True, default=uuid.uuid4) 1499 | username: Mapped[str] = mapped_column(String(50), unique=True, nullable=False) 1500 | hashed_password: Mapped[str] = mapped_column(String(255), nullable=False)

// ====== Page 30 ======